How to install/upgrade

Topic in this document

How to install/upgrade the linear plugin v1.2	1

# How to install/upgrade the linear plugin v1.2

This patch is applicable in all SC-DAD devices with analog inputs (from version v2.0 to v2.4). NOTE: The installation/upgrade will need to reboot the device, the data acquisition will be stopped for 1 minute.

To apply the patch

1) Disable the linear plug-in if enabled (by selecting the raw plug-in instead of the linear plug-in from the Ai data treat panel in the Input Configuration section), reboot the device.

2) Click on the Tools menu button from the web configuration software (to access on the web configurator open a web browser and type: http://xxx.xxx.xxx.xxx/ where xxx.xxx.xxx.xxx is the SC-DAD IP)

3) Insert the system password if asked (default password: root) and then click to the login button

4) Click on the Software Upgrade/Backup Restore button

5) Select the linear 1.2 file and use “send” button to start the upgrade

A white page will appear, then wait for the device automatic reboot (15 seconds) and then close the web page.

NOTE: Don't use the refresh or back button of the web browser.

Wait 30 seconds and open the web configurator  again (to access on the web configurator open a web browser and type: http://xxx.xxx.xxx.xxx/ where xxx.xxx.xxx.xxx is the SC-DAD IP) open the linear plug-in configuration page and check the version on the top of the configuration page (it should be v1.2).